- Install onto EBP Sensor. The two shorter cables connect in line between the trucks connectors. Plug the “female” end into the sensor, and the “male” end into the wiring harness. See Figure 1

- Route the longer cable down to the pressure sensor. Run the cable back, and then down along the dip stick, and then rearward to the sensor. See Figure 2

- Install onto pressure sensor. Unplug pressure sensor & plug cable in place of pressure sensor.

- Secure any loose or sagging cable with clips or zip ties as needed.

- Clip the temp sensor emulators onto the temp sensor wiring harnesses. The front temp sensor is in front of the test pipe. Unplug the front temp sensor and plug in a temp sensor emulator.

- Optional: use the included heat shrink on the OEM sensor plug to keep it clean in case you return the truck back to stock in the future

Figure 1

Figure 2

Figure 3

Figure 4

Figure 5

Figure 6

Figure 7